Design and Implementation of a Quadcopter

By

Marcos Negreira

Felipe Soares

GROUP 18

Date: 	December, 2015

Professor:  Dr. Bassem Alhalabi,

Table of Contents

1    Introduction	3

# 1    Introduction

In this  project,  a QuadCopter is designed and  implemented to autonomously fly and  demonstrate on-board  GPS processing capabilities.  The project is aimed to control the QuadCopter by virtue of two control systems; the local control,  and the global control.  The local control is intended  for the stabilization of the QuadCopter, so that it is able to fly manually  through  the RC receiver and  the  global control  is intended  to make the  QuadCopter to fly autonomously with the aid of control  through  the GPS.  These two control  loops are implemented on dedicated  processors with appropriate capabilities.

# 2    System  Architecture

In this section, we will discuss the used hardware  and system architecture. First  we will take a look at  the  local control,  global control  and  used sensors,  and  after  that, we will address communication used between  these components.

###### 2.1    Local Control

The local control is powered by  CC3D development board (figure 1), powered by the ~CC3D~ processor 16 MB flash memory.  This board also contains a gyroscope, accelerometer  and a magnetic  sensor, all measuring  three  axis.  This makes it an ideal board for a quadcopter’s  local control,  because it contains  enough processing power, and has all the basic sensors available. A SPEKTRUM AR610 was used for testing manually.

Figure  1: An CC3D.

###### 2.2    Global Control

For  the  global control,  we have  chosen  for the  Raspberry PI  (figure 2).   We chose for this platform   because  it  was  cheap,  we already  had  it  available,  and  the camera  module  was expected  soon after the start of the project.

Figure  2: A Raspberry PI.

###### 2.3    Sensors

The  local control  board  already  contained  the  basic sensors required  to fly a quadcopter: a gyroscope, an accelerometer  and a magnetic  sensor.  To this list, we added a battery monitor.

###### 2.3.1     Battery  monitor

The battery monitor was a simple voltage divider that connected  the battery voltage to one of the ADC pins of the CC3D  board.  The voltage divider was necessary to convert the voltage to the 0 – 3V domain.

###### 2.3.2     Ultrasound

Initially  the plan was to use the  ultrasound sensor (figure 3) for tracking  the altitude of the quadcopter.  The  HCSR04 sonar  driver  was requested  from Taulabs development  team  and merged to our code base.  The ultrasound driver implementation could be obtained  in Github

link1.   As we further  realized,  the  ultrasound sensor  suffered propeller  sound  interference which made the sensor values noisy and inaccurate. This happened because there were high frequency  components  in the  sound  waves emitted  by the  propellers  and  it fell in the  band of 40 KHz which was the operating  frequency of the sound waves used for ultrasound sensor, hence  this  interference  resulted  in changing  our  design  decision to  use alternative altitude tracking  sensor such as barometer instead  of the ultrasound sensor.

Figure  3: The HCSR04 sensor.

1 https://github.com/mnegreira/TauLabs

###### 2.3.4     Cameras

We have chosen for the Raspberry PI cameras,  because they  are affordable,  of relative  good quality,  small and  light,  and  they  uses few system  resources  on the  Raspberry PI.  Because each Raspberry PI  can only interface  with  one camera. (This was not part of the Scope of the project)

These  cameras  are  able to  film in 1080p resolution  with  30 fps, and  take  still images with

2592 x 1944 resolution.

Figure  5: Raspberry PI with camera  module.

###### 2.4    Communication

There  are many  communication-paths, as can be seen in figure 6.  In this  section,  these  will all be discussed.

Figure  6: Overview of different communication protocols.

Figure 6.1: Overview of different communication protocols.

###### 2.4.1     Local control and global control

The local control on the CC3D  communicates with the first Raspberry PI using an USB-cable.  They  use the  UAVTalk2  protocol,  and  thus  communicate high level objects. Because  the  UAVTalk  protocol  is already  seamless integrated in the  local control  (it  is also used for communication with the Ground  Control  Station), all useful pieces of data  are avail- able as a UAVObject.

This  makes it an extremely  powerful interface.   Global  control  algorithms  have access to all the  information  that it  available  to  the  local control,  and  it  can  influence and  override  all aspects  of the local control  by writing  new values to these objects.

The  interface  is also flexible.  Currently it uses USB, but  UART  or other  protocols  are also possible.   The  UAVObjects  on the  local control  are  also accessible on a laptop  and  on the other  Raspberry PI, using a WiFi or an ethernet connection,  thus eliminating  the need for a separated (bluetooth) telemetry module.

###### 2.4.2     Laptop

On the  Raspberry PI  runs  a normal  Linux  installation, and  that gives the  option  to add  a WiFi  adaptor so that it can communicate with  the  outside  world using WiFi.   We did this, and  even shared  the  internet connection  to  the  second  Raspberry PI,  so both  could  easily download and install  new software.

2 http://wiki.openpilot.org/display/Doc/UAVTalk

###### 2.4.5     Camera

The cameras  are attached to the  Raspberry PIs,  and  use a flatcable  for the  connection  to a

CSI3-interface.

3    Software

To get all this hardware  flying, we need some software to control everything. We have software for local and global control,  and both  will be discussed in this section.

###### 3.1    Local control

For the  local control,  we chose for the  open source project  TauLabs4 .  This  project  is a fork from the OpenPilot project.  We choose for this project,  because is was already  ported  to the hardware  we where using. Also a SPEKTRUM AR610 was used for control.

This  project  provided  not  only  software  for  the  local  control,   there  was  also  a  “Ground Control  Station” (GCS).  This  program  runs  on your computer and  communicated with  the local control  to provide configuration  options  and means of debugging  and monitoring.

###### 3.2    Global control

We have written  our own global control program.  This program  runs on a Raspberry PI, and is a stripped  down version of the GCS provided  by TauLabs. This allows it to communicate with the local control  using UAVObjects, as is discussed in section 2.4.1.  Our global control code is also available  on GitHub5.

The  global control  has a fixed core, and  allows functionality to  be added  by modules.   We did not  have enough time  to write  really interesting modules,  but  we do have some “proof- of-concept”  modules,  which will be discussed next.  These plugins show that it is possible to read and write data  from/to the local control.

###### 3.2.1     Recorder plugin

This plugin monitors  the “Armed”  flag in the “FlightStatus” UAVObject, and as soon as the quadcopter gets armed,  it starts recording  video on both  Raspberry PIs.  On disarming,  the recording  stops.  This allows us to easily create  in-flight footage with the cameras.

3 Camera Serial  Interface

4 https://github.com/TauLabs/TauLabs

5 https://github.com/mnegreira/TauLabs

4    Objectives

Here we will discuss the objectives  given in the course, and how we handled  them.

###### 4.1    Building Quadcopter  and flying

We have build our quadcopter, and made fly. See figure 7 for a photo.

Figure  7: Our quadcopter.

###### 4.2    Tuning Local control

We spend a lot of time on PID tuning,  but only in the end we found that some of the settings where completely wrong; the direction  that the motors where spinning was configured wrong, so yaw corrections  resulted  in more  yaw,  crashing  the  quadcopter.  Also one of our  speed controllers  was not functioning  correctly,  which led to an unstable  quadcopter.

###### 4.3    Altitude  hold

Altitude hold should have been possible with the barometer installed,  but  there  was no time to actually  test  this.

###### 4.4    Autonomous  flight

We did not manage to get the quadcopter fly autonomously, because it was not stable enough and still needed some manual  corrections,  and we didn’t  have altitude hold working.

###### 4.5    Drift correction in horizontal axis

We where unable  to implement  this,  because  we didn’t  have altitude hold and  autonomous flight.

B 	Partlist

Here follows a list  of all the  parts  and  components  that we have  used  while building  this quadcopter.